### Information Technology:

### Digital Images

There are three ways to create digital images: scan a conventional photographic print, scan a 35-mm slide, or capture the image with a digital camera (either digital video camcorder or digital still camera).

| - Table 1. - Comparison of methods of capturing digital Images |  |  |  |
| --- | --- | --- | --- |
|  | Digitally scan 35-mm slide | Digitally scan 35-mm prints | Photograph with digital camera |
| step 1 | Setup and photograph using 35-mm camera and slide film | Setup and photograph using 35-mm camera and print film | Setup and photograph using digital camera |
| step 2 | Process the slides (1 to 3 days) | Process the prints at a one hour photo processor. | Transfer digital image to computer (2 minutes) |
| step 3 | Scan the images with a slide scanner (5 minutes) | Scan the images with a flat bed scanner (5 minutes) |  |
| step 4 | Transfer digital image to computer (2 minutes) | Transfer digital image to computer (2 minutes) |  |
| Equipment cost | $300 (and up) digital slide scanner + 35-mm equipment | $100 flatbed scanner + 35-mm equipment | $500-700 for digital camera |
| Average time per image | 15 minutes + time to get film processed | 15 minutes + time to get film processed | 5 minutes |

The final use of the images will determine which method is best. If the images will ever be used to make a presentation to a large group, then using 35-mm slides may still be the best way to make the initial image because they are much higher resolution. It is true, that digital projectors are becoming more prevalent and it is also possible to create slides from digital images. But it is advisable to start with 35-mm then scan to create digital images (high resolution to low resolution), rather than the other way around.

If the images will be reproduced in a catalog or book someday and the final image size will be larger than 3" then 35-mm may also be the best way to make initial image because of better photographic quality.

If you already have a large volume of existing photographs, 4x6" prints for example, then scanning the images with a flatbed scanner will be the easiest way to transfer the images to digital format.

However, if the final application is posting images to an online auction or e-commerce site, and a high volume of photos is anticipated, then capturing the images with a digital camera will be more cost effective and less time consuming.

The bottom line is the trade off between quality, cost and convenience. If you anticipate 100 images per year then either 35-mm slides or prints will suffice. If you anticipate shooting 50 images a week then convenience and time savings will be more important than the higher cost of a digital camera. The task of photographing a mineral collection of say 1000 specimens is not insignificant. Obviously, it pays to buy the best equipment for the job. It will save time, effort and ensure image quality. For the purposes of this article, focus will be on using a digital still camera, the most likely method used by mineral collectors/photographers. Much of the same applies to using digital video camcorders.

### Camera Resolution

Digital image resolution is measured in pixels (short for Picture Elements). A pixel is an individual point of color in an image and is the equivalent to the film grain in conventional photography. It is expressed as a length x width number. Cameras are often sold as megapixel cameras meaning they capture one million pixels of information. The length multiplied times the width yields the total pixel count.

#### Pixels vs. DPI

Image pixels are independent of the common specification used in printing known as DPI for dots per inch (or LPI for lines per inch). DPI/LPI have no relation to the image quality captured by the camera. They are a control of the printer and the size of the print image. As an example, a 600 x 300 pixel image printed at 300 DPI will output a 2x1" image on paper. The same 600 x 300 pixel image printed at 150 DPI will produce a 4x2" image. The content captured by the camera (600 x 300 pixels or 180,000 total pixels) and stored as the digital image is exactly the same in both cases, even the file size is the same. But the print size will be different. Print resolution of 300 DPI is minimum before print quality starts to degrade, 600 DPI is common on laser printers, and 1200 DPI is readily available.

The pixel count does effect how large an image is printed before there is apparent degradation. DPI has no effect on the inherent image quality of the digital image. This a major source of confusion and important to understand. The bottom line is the important measure of image quality is pixel count expressed as length x width ( i.e. 600 x 300 pixels). See Table 2. for the file and print size comparisons.

As a point of reference, a 35-mm Kodachrome slide captures the equivalent of about 1 billion pixels of information, which is why Fuji and Eastman Kodak is not too worried that conventional photography will ever disappear.

| - Table 2. - A summary of typical image and camera specifications and their equivalents: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Typical image resolutions: |  | Total pixel count: |  | Camera advertised as: | Typical JPEG file size: | Printed image size at 150 DPI: | Printed image size at 300 DPI: |
| 640 x 480 pixels | = | 307,200 pixels | = | 300k pixels | 130 kb | 4.2x3.2" | 2.1x1.6" |
| 1024 x 768 pixels | = | 786,432 pixels | = | 786k pixels | 260 kb | 6.8x5.2 | 3.4x2.6" |
| 1280 x 1024 pixels | = | 1,310,720 pixels | = | 1.3 megapixels | 520 kb | 8.6x6.8" | 4.3x3.4" |

#### How Many Pixels Do I Need?

The answer is simple: as many as you can afford. The price of digital cameras is directly related to the resolution, and therefore image quality.